AgriSat AI Drip Irrigation System

Comprehensive Technical Report

CAET Campus, Dediapada  •  21.628°N, 73.592°E  •  Navsari Agricultural University

| System Type | Coverage | Cycle | Status |
| --- | --- | --- | --- |
| AI Satellite Drip | 3 Campus Zones | Every 3 Days | Autonomous |

# 1. System Overview

AgriSat is an end-to-end, AI-driven precision irrigation platform specifically designed for the CAET campus at Dediapada. The system autonomously fuses satellite imagery, real-time weather intelligence, and soil physics to determine when and how much water to apply to each field zone — with zero manual intervention.

The system operates on a 3-day autonomous cron cycle. During each cycle, it:

- Acquires the latest cloud-free Sentinel-2 satellite scene over Dediapada

- Computes per-zone NDVI (Normalized Difference Vegetation Index)

- Fetches live weather data including ET₀, rainfall forecast, and VPD

- Applies a multi-layer decision logic to assign an irrigation status to each zone

- Issues valve commands and calculates the precise irrigation volume required

- Logs all decisions to CSV and Google Sheets for traceability

- Uploads NDVI maps and zone JSON reports to Google Drive

# 2. System Architecture — Three Layers

## Layer 1: Satellite NDVI Engine  (ndvi_harvester.py)

Queries Microsoft Planetary Computer for the latest cloud-free Sentinel-2 L2A scene over Dediapada. Key operations:

- Computes NDVI = (Band8_NIR − Band4_Red) / (Band8_NIR + Band4_Red)

- Generates a colourised JPEG map (red = bare/stressed, green = healthy vegetation)

- Clips NDVI to 3 named campus zones using saved GeoJSON boundaries (Shapely + Rasterio)

- Runs on a 3-day automatic cron cycle

- Uploads outputs to Google Drive via Selenium (incognito Chrome)

## Layer 2: Weather Intelligence  (get_weather_context + Open-Meteo)

Pulls live meteorological data from the Open-Meteo API (free, no API key required):

| Parameter | Source | Purpose |
| --- | --- | --- |
| ET₀ (mm/day) | Open-Meteo | Evapotranspiration rate — crop water loss |
| 3-day Rain Forecast (mm) | Open-Meteo | Prevents unnecessary irrigation before rain |
| VPD (kPa) | Open-Meteo | Vapour Pressure Deficit — crop stress indicator |

Weather Override Rules:

| Condition | Override Action |
| --- | --- |
| STRESSED + Rain ≥ 15 mm forecast | → HOLD (skip irrigation) |
| HEALTHY/THRIVING + ET₀ > 8 mm/d | → MODERATE (irrigate sooner) |
| MODERATE + VPD > 3.0 kPa | → STRESSED (irrigate immediately) |

## Layer 3: Drip Decision Dashboard  (app.py — Streamlit)

- Monitors 3 field zones (A, B, C) with live soil VWC (Volumetric Water Content)

- Calculates crop water demand using FAO Penman-Monteith ET₀ × Kc (crop coefficient)

- Drip system specs: 3 mm/hr application rate at 90% efficiency

- Fetches EOS-04 SAR data from ISRO Bhoonidhi API for radar-derived soil moisture

- Logs every irrigation decision to Google Sheets via service account

# 3. Soil Water Balance Model

The system uses a Soil Water Balance approach to determine crop water status and refill requirements.

| Parameter | Value | Significance |
| --- | --- | --- |
| Field Capacity (FC) | 32% | Maximum water soil can hold |
| Wilting Point (WP) | 14% | Minimum water before crop wilts |
| Total Available Water (TAW) | 18% | FC − WP = water available to crop |
| Refill Trigger (VWC) | 23% | WP + 50% × TAW — irrigation starts here |

# 4. Key Equations

| NDVI | NDVI = (Band8_NIR − Band4_Red) / (Band8_NIR + Band4_Red) |
| --- | --- |

| Soil Water Balance | Net VWC = Previous VWC + Rain − ET₀ × Kc − Drainage |
| --- | --- |

| Irrigation Volume | Vol (L) = (FC − VWC) × root_depth_m × area_m² ÷ drip_efficiency |
| --- | --- |

| Reference ET₀ | ET₀ = f(solar radiation, temperature, humidity, wind speed) [FAO Penman-Monteith] |
| --- | --- |

# 5. NDVI Decision Thresholds

| NDVI Range | Status | Action | Timeframe |
| --- | --- | --- | --- |
| < 0.25 | STRESSED | Irrigate | Within 24 hours |
| 0.25 – 0.35 | MODERATE | Irrigate | In 2–3 days |
| 0.35 – 0.50 | HEALTHY | Hold | Normal schedule |
| > 0.50 | THRIVING | No action | Skip cycle |

# 6. Technologies & Components

| Component | Technology | Notes |
| --- | --- | --- |
| Satellite Imagery | Microsoft Planetary Computer | Sentinel-2 L2A — free, cloud API |
| Radar Soil Moisture | ISRO Bhoonidhi API | EOS-04 SAR C-band radar |
| Weather Data | Open-Meteo API | Free, no API key required |
| Dashboard | Python + Streamlit | Live zone monitoring |
| Zone Geometry | GeoJSON + Shapely + Rasterio | Campus boundary clipping |
| Drive Upload | Selenium (Chrome incognito) | Automated NDVI map upload |
| Data Logging | Google Sheets API | Service account, remote access |
| Map Display | Folium | Interactive zone maps |

# 7. Output Files Per Cycle

| File / Output | Contents |
| --- | --- |
| NDVI_Dediapada_scene-[date]_downloaded-[date].jpeg | Colourised NDVI satellite map of campus |
| NDVI_ZoneReport_scene-[date]_downloaded-[date].json | Per-zone NDVI, status, action, weather context |
| telemetry_log.csv | All irrigation decisions: timestamp, VWC, ET₀, valve state |
| Google Sheets (live) | Same telemetry, remotely accessible, auto-updated |

# 8. System Decision Flowchart

The flowchart below illustrates the complete autonomous decision pipeline — from satellite data acquisition through valve command and logging, repeating every 3 days.

# 9. Decision Logic — Per Zone

Each zone (A, B, C) processes independently through the following sequential decision pipeline:

- Step 1 — NDVI Threshold: Raw satellite NDVI value is compared against 4 thresholds to assign a base status (STRESSED / MODERATE / HEALTHY / THRIVING)

- Step 2 — Weather Override: Live weather data (ET₀, rain forecast, VPD) is checked and may upgrade or downgrade the base status

- Step 3 — Final Status: Resolved zone status: STRESSED / MODERATE / HEALTHY / THRIVING / HOLD

- Step 4 — Valve Command: STRESSED or MODERATE → Open (irrigate); HEALTHY / THRIVING / HOLD → Keep closed

- Step 5 — Volume Calculation: Vol (L) = (FC − current VWC) × root_depth_m × area_m² / drip_efficiency

- Step 6 — Logging: Decision written to telemetry_log.csv and Google Sheets with timestamp

# 10. Operational Status

| Field | Value |
| --- | --- |
| Location | CAET Campus, Dediapada — 21.628°N, 73.592°E |
| Institution | College of Agricultural Engineering & Technology, Navsari Agricultural University |
| Report Prepared | June 2026 |
| Next Autonomous Cycle | 2026-06-09 |
| System Mode | Fully Autonomous — 3-day cron |
| Zones Active | A, B, C (3 campus field zones) |
| Data Sources | Sentinel-2 L2A + ISRO EOS-04 SAR + Open-Meteo |

This report was auto-generated from the AgriSat system design specification. All equations, thresholds, and logic are derived directly from FAO-56 Penman-Monteith guidelines and standard remote-sensing agronomic practice.